RFID Attendance System ESP8266 Firmware Developer Guide Arduino ¦ MFRC522 ¦ Firebase ¦ Version 1.0

⚠️ Get real Firebase credentials from /config/firebase_config.js in the GitHub repo before flashing any firmware.

# 1. Your Role

Your job is to program the ESP8266 + MFRC522 reader to: detect RFID taps, look up whether the tag is authorised, and write a structured log entry to Firebase Realtime Database. The database and dashboard are already set up — you just need to write correctly formatted data to /logs.

# 2. Hardware & Wiring

MFRC522 to ESP8266 NodeMCU pin mapping:

| MFRC522 Pin | ESP8266 NodeMCU Pin |
| --- | --- |
| SDA (SS) | D8 (GPIO15) |
| SCK | D5 (GPIO14) |
| MOSI | D7 (GPIO13) |
| MISO | D6 (GPIO12) |
| GND | GND |
| RST | D3 (GPIO0) |
| 3.3V | 3V3 |

Important: MFRC522 runs on 3.3V — do NOT connect to 5V or you will damage the module.

# 3. Required Libraries

Install all three via Arduino IDE → Sketch → Include Library → Manage Libraries:

| Library | Author | Search Term |
| --- | --- | --- |
| Firebase ESP Client | Mobizt | Firebase ESP Client |
| MFRC522 | GithubCommunity | MFRC522 |
| ArduinoJson | Benoit Blanchon | ArduinoJson |

# 4. Firebase Credentials

At the top of your sketch define these — get the real values from /config/firebase_config.js in the repo:

#define WIFI_SSID "your_wifi_name" #define WIFI_PASSWORD "your_wifi_password" // From firebase_config.js #define API_KEY "YOUR_API_KEY" #define DATABASE_URL "https://attendance-logging-syste-5540c-default-rtdb.europe-west1.firebasedatabase.app" // Identify this reader — change per device #define READER_ID "reader_01" #define LOCATION "Building A — Main Entrance" #define DEPARTMENT "Computer Science" #define EVENT_TYPE "entry" // 'entry' for entrance readers, 'exit' for exit readers

# 5. Full Firmware Sketch

Save this as esp8266_main.ino inside /firmware/esp8266_main/ in the repo:

#include <Arduino.h> #include <ESP8266WiFi.h> #include <Firebase_ESP_Client.h> #include <SPI.h> #include <MFRC522.h> // ── Pin definitions ────────────────────────────────────── #define SS_PIN D8 #define RST_PIN D3 // ── Credentials (replace with values from firebase_config.js) ── #define WIFI_SSID "your_wifi" #define WIFI_PASSWORD "your_password" #define API_KEY "YOUR_API_KEY" #define DATABASE_URL "https://attendance-logging-syste-5540c-default-rtdb.europe-west1.firebasedatabase.app" #define READER_ID "reader_01" #define LOCATION "Building A" #define DEPARTMENT "Computer Science" #define EVENT_TYPE "entry" MFRC522 rfid(SS_PIN, RST_PIN); FirebaseData fbdo; FirebaseAuth auth; FirebaseConfig config; void setup() { Serial.begin(115200); SPI.begin(); rfid.PCD_Init(); // Connect to WiFi WiFi.begin(WIFI_SSID, WIFI_PASSWORD); Serial.print("Connecting to WiFi"); while (WiFi.status() != WL_CONNECTED) { delay(300); Serial.print("."); } Serial.println(" Connected! IP: " + WiFi.localIP().toString()); // Connect to Firebase config.api_key = API_KEY; config.database_url = DATABASE_URL; Firebase.signUp(&config, &auth, "", ""); // anonymous auth Firebase.begin(&config, &auth); Firebase.reconnectWiFi(true); Serial.println("Firebase ready"); } void loop() { if (!rfid.PICC_IsNewCardPresent() ¦¦ !rfid.PICC_ReadCardSerial()) return; // Build tag UID string String tagUid = ""; for (byte i = 0; i < rfid.uid.size; i++) { if (rfid.uid.uidByte[i] < 0x10) tagUid += "0"; tagUid += String(rfid.uid.uidByte[i], HEX); } tagUid.toUpperCase(); Serial.println("Tag detected: " + tagUid); // Build JSON log entry FirebaseJson json; json.set("tagUid", tagUid); json.set("readerId", READER_ID); json.set("eventType", EVENT_TYPE); json.set("timestamp/.sv", "timestamp"); // Firebase server time json.set("status", "granted"); // TODO: check Firestore for real auth json.set("location", LOCATION); json.set("department", DEPARTMENT); // Write to /logs in Realtime Database if (Firebase.RTDB.pushJSON(&fbdo, "/logs", &json)) { Serial.println("Log written: " + tagUid); } else { Serial.println("Write failed: " + fbdo.errorReason()); } rfid.PICC_HaltA(); rfid.PCD_StopCrypto1(); delay(1500); // debounce — prevent double-tap }

# 6. Required Log Format

Every entry your firmware writes to /logs MUST include these fields exactly as named. The dashboard and analytics depend on them:

| Field | Type | Example | Required |
| --- | --- | --- | --- |
| tagUid | string | A1B2C3D4 | Yes — tag hex UID |
| readerId | string | reader_01 | Yes — identifies the device |
| eventType | string | entry | Yes — 'entry' or 'exit' |
| timestamp | server ts | auto | Yes — use .sv: timestamp |
| status | string | granted | Yes — 'granted' or 'denied' |
| location | string | Building A | Yes — physical location |
| department | string | Computer Science | Yes — for analytics grouping |
| userName | string | Alice Mwangi | Optional — resolve from Firestore |
| role | string | student | Optional — resolve from Firestore |

# 7. Testing Your Setup

Follow these steps in order to verify everything works before integrating with the full system:

| Step | What to do |
| --- | --- |
| 1 | Upload WiFi test sketch. Serial Monitor should print IP address. |
| 2 | Upload full sketch. Tap any RFID card. Serial Monitor should print 'Log written: XXXXXXXX'. |
| 3 | Go to Firebase Console → Realtime Database. A new entry should appear under /logs instantly. |
| 4 | Verify all 7 required fields are present in the log entry. |
| 5 | Tap the same card again — a second entry should appear with a different push ID. |

Serial Monitor baud rate must be set to 115200 to match Serial.begin(115200) in the sketch. If you see garbled characters, check the baud rate dropdown at the bottom of Serial Monitor.

# 8. Deploying Multiple Readers

For each physical reader deployed, change these three defines before flashing:

// Entry reader — Building A #define READER_ID "reader_01" #define LOCATION "Building A — Main Entrance" #define EVENT_TYPE "entry" // Exit reader — Building A #define READER_ID "reader_02" #define LOCATION "Building A — Exit" #define EVENT_TYPE "exit" // Entry reader — Admin Block #define READER_ID "reader_03" #define LOCATION "Admin Block — Entrance" #define EVENT_TYPE "entry"

The reader IDs must match the document IDs in the Firestore /readers collection. Ask Davis to confirm the IDs before flashing.

# 9. Troubleshooting

| Problem | Fix |
| --- | --- |
| Serial Monitor shows garbled text | Set baud rate to 115200 in Serial Monitor |
| WiFi not connecting | Double-check SSID and password. 2.4GHz only — ESP8266 does not support 5GHz |
| Write failed: connection refused | Check DATABASE_URL is correct and matches firebase_config.js |
| Write failed: permission denied | Anonymous auth not initialised. Ensure Firebase.signUp() runs in setup() |
| Tag not detected | Check SPI wiring. Confirm SS_PIN is D8 and RST_PIN is D3 |
| Double-tap logging same card twice | Increase delay() at end of loop. Currently 1500ms |
| Upload error: PermissionError(13) | Wrong COM port or missing CH340/CP2102 driver. Check Device Manager |

Questions? Open an issue on the GitHub repo or message the group chat. For Firebase Console access ask Davis to add your Google account under Project Settings → Users and Permissions.